Project Proposal — ESP32 Light‑Reactive Automation System

Project Overview

This project is a light‑reactive automation system built using an ESP32 microcontroller, a light sensor (LDR), and two indicator LEDs. The ESP32 continuously measures ambient light levels and communicates those readings to a host Python script running on a PC. The host script receives the live sensor data over a serial connection, logs it, and can be extended to trigger additional automated actions.

The goal of this project is to demonstrate a complete automation pipeline that integrates hardware, firmware, serial communication, and host‑side processing, fulfilling the core requirements of the Automation Capstone.

Problem Statement

Light levels in a room or workspace can change throughout the day, affecting visibility, comfort, and energy usage. This project provides a simple but effective automated monitoring system that reacts to environmental changes and reports them to a computer for logging or further automation. The system can be expanded into a smart‑home lighting controller, an environmental monitor, or a data‑collection tool.

Key Components

ESP32 DevKit running MicroPython

LDR (photoresistor) for sensing ambient light

10 kΩ resistor for voltage‑divider input

Two LEDs (bright/dark indicators)

220 Ω resistors for LED current limiting

Host computer running a Python script

USB serial connection for communication

GitHub repository for version control and documentation

Core Functionality

Hardware Interfacing

The LDR measures ambient light levels. The ESP32 reads the analog value and uses two LEDs to indicate whether the environment is bright or dark.

Microcontroller Programming

The ESP32 firmware (MicroPython) performs ADC sampling, threshold comparison, LED control, and serial output of sensor values.

Communication Protocol

The ESP32 sends live sensor readings to the host computer using serial communication (UART over USB).

Host Interaction

A Python script on the PC receives the serial data, prints it in real time, and can log or react to the readings. This demonstrates integration between hardware and a host‑side automation process.

Version Control

All project files—including firmware, host script, schematic, and documentation—are tracked in Git and pushed to a GitHub repository.

Intended Outcome

The final system provides a functional demonstration of environmental sensing, microcontroller control, and PC‑side automation. It serves as a foundation for more advanced projects such as smart lighting, environmental monitoring dashboards, or automated alert systems.